| 150WLED BEAM | 14CH Mode1 | 13CH Mode2 | DMX Value | Funciton |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Darkness----->Bright |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open Slow Pulsation--->Fast Pulsation Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 | 0---4 5---8 9---12 13---17 18---21 22---55 26---29 30--- 34 35--- 38 39---42 43---46 47---51 52---55 56---59 60---63 64---68 69---72 73---76 77---81 82---85 86---89 90---93 94---98 99---102 103---179 180---255 | WHITE WHITE +Color1 Color1 Color1 +Color2 Color2 Color2 +Color3 Color3 Color3 + Color4 Color4 Color4 + Color5 Color5 Color5+Color6 Color6 Color6 +Color7 Color7 Color7 + Color8 Color8 Color8 + Color9 Color9 Color9 +Color10 Color10 Color10 + Color11 Color11 Color11 + WHITE CW SlowRotation---->Fast Rotation CCW Slow Rotation---->Fast Rotation |
| Gobo1(Fixed Gobo) | 5 | 4 | 0---3 4---7 8---11 12---15 16---19 20---23 24---27 28---31 32---35 36---39 40---43 44---47 48---51 52--- 55 56---59 60---63 64---67 68---71 72---113 114---117 118---159 160---166 167---172 173---179 180---185 186--- 191 192---198 199---204 205---211 212---217 218---223 224---230 231---236 237---243 244---249 250---255 | WHITE GOBO 1 GOBO 2 GOBO 3 GOBO 4 GOBO 5 GOBO 6 GOBO 7 GOBO 8 GOBO 9 GOBO 10 GOBO 11 GOBO 12 GOBO 13 GOBO 14 GOBO 15 GOBO 16 GOBO 17 CW SlowRotation------>FastRotation WHITE CCW SlowRotation------> FastRotation GOBO 2 Shake Slow---->Fast GOBO 3 Shake Slow---->Fast GOBO 4 Shake Slow---->Fast GOBO 5 Shake Slow---->Fast GOBO 6 Shake Slow---->Fast GOBO 7 Shake Slow---->Fast GOBO 8 Shake Slow---->Fast GOBO 9 Shake Slow---->Fast GOBO10 Shake Slow---->Fast GOBO11 Shake Slow---->Fast GOBO12 Shake Slow---->Fast GOBO13 Shake Slow---->Fast GOBO14 Shake Slow---->Fast GOBO15 Shake Slow---->Fast GOBO16 Shake Slow---->Fast |
| PRISM | 6 | 5 | 0---127 128--255 | EXCLUDED INSERTED |
| PRISM Rotation | 7 | 6 | 0---127 128---190 191---194 195---255 | Position 0°---->360° CW FastRotation------> SlowRotation STOP CCW Slow Rotation------>Fast Rotation |
| FOCUS | 8 | 7 | 0---255 |  |
| PAN(X) | 9 | 8 | 0---255 |  |
| PAN(X) FINE | 10 | 9 | 0---255 |  |
| TILTE(Y) | 11 | 10 | 0---255 |  |
| TILTE(Y) FINE | 12 | 11 | 0---255 |  |
| P/T SPEED | 13 | 12 | 0---128 129---170 171---211 212---255 | Fastest------->Slowest Slow Speed ( 1/3 MaxSpeed) Medium Speed ( 1/2 MaxSpeed) Fast Speed (Fastest Speed) |
| Control | 14 | 13 | 0---127 128--255 | Undefined Reset ALL |